- 1. The guards protecting the working point, the two-hand control device, and any photocell device shall not be modified, disabled, or taken out of service.

- 2. Adjustment, maintenance, and repair work on presses shall only be carried out by authorized personnel, with all energy connections disconnected.

- 3. When pressing small parts, the parts shall be held with special clamps, pliers, or tongs and fed into the press.

- 4. Before installing dies on mechanical presses, the motor shall be stopped. For hydraulic and pneumatic presses, the pressure connection shall be disconnected, and sufficiently strong blocks shall also be placed between the press head and the table.

- 5. Presses shall be operated within their normal capacity and in accordance with their specifications.

- 6. Double-stroking of presses for any reason shall be prevented, and the necessary corrections shall be made to prevent this condition.

- 7. Sufficient clearance shall be maintained around the operator to allow safe and comfortable operation.

- 8. All guards required for the operation being performed shall be in place and in proper working condition.

- 9. Personnel other than maintenance and repair staff shall be prohibited from interfering with the clutch, brake pedal, or starting mechanism.

- 10. The press shall be stopped when lubrication, adjustment, or repair is required, or whenever the operator must leave the workstation during operation.

- 11. After stopping and leaving the machine and upon returning to work, the machine shall be checked again to ensure that it is operating safely.

- 12. Situations or activities that may distract the press operator shall be prevented.

- 13. The press shall not be operated by unqualified or inadequately trained personnel.

ACKNOWLEDGED BY						NOTIFIED BY

Full Name:								Full Name:

Signature:								Signature:

Date:									Date: