1. Do not perform drilling without securely fastening the drill bit.

2. Check whether the drill press is in suitable working condition.

3. Make sure that the vise is securely fastened to the table.

4. Always wear safety goggles and use a face shield against flying chips.

5. Do not attempt to remove chips with your hands. Use a hook if necessary.

6. Do not leave the drill press unattended while it is operating.

7. Keep your head away from the rotating spindle.

8. For safe operation of the drill press, do not touch the rotating drill bit with your hands. Do not wipe it with a cloth or waste cloth. Only a brush shall be used for cleaning the drill press.

9. Do not drill without securely fastening the workpiece to the table or drill press vise.

10. Do not drill with a dull drill bit.

11. Do not drill without securing the machine table.

12. When drilling, make sure there is sufficient clearance underneath the workpiece.

13. Do not touch rotating parts.

14. Make sure that no object comes into contact with rotating parts.

15. After adjusting the speed, close the belt guard. Do not operate the machine while the guard is open.

16. Use coolant during the drilling operation.

17. Stack drilled parts in a location where they will not interfere with your work.

18. Lubricate the parts that require lubrication. Check the oil level periodically to prevent the machine from operating without sufficient lubrication.

19. For safe operation of the drill press, stop the machine when the drilling operation is complete. Do not stop the chuck by hand.

ACKNOWLEDGED BY							NOTIFIED BY

Full Name:								Full Name:

Signature:								Signature:

Date:									Date: