Full compliance with the requirements specified in the project and specifications is essential in all applications.

Brick and Blockwork

- Ensure that the mortar thickness is neither less nor greater than required.

- Horizontal and vertical joints shall be of the same thickness, and vertical joints shall be staggered.

- Bricks and blocks shall have a low water absorption rate.

- If the wall surface will be plastered, the joints shall be flush with the surface; if it will not be plastered, they may be recessed or protruding.

- In hot weather, the wall surfaces shall be wetted periodically after completion to prevent mortar cracking and cracks caused by water loss.

- The wall surface shall be kept level and plumb to reduce plaster waste.

Plastering Works

- Plaster shall be prepared from a mixture of water, cement, plaster sand, and lime. The lime shall be fully slaked, and the sand shall be neither too fine nor too coarse (of the required grading).

- Before plastering begins, the wall shall be divided into screed sections. The sections shall be marked with manually applied vertical strips of plaster 20–25 cm wide.

- Plaster thickness shall be checked frequently.

Painting Works

- The paints specified in the project/specifications shall be used, and trial applications in alternative colours shall be made and submitted to the supervising engineer for approval.

- Spray painting on exterior surfaces shall be avoided in windy weather.

Cladding Works

- Cladding works include covering elements such as walls, roofs, and floors with prefabricated building components.

- Application/detail drawings shall be prepared to ensure high-quality cladding and minimize waste.

- Attention shall be paid to joint details to ensure visual integrity.

- Detailed drawings shall be prepared for sections where different claddings meet, and potential problems likely to arise during application shall be resolved at this stage.

Waterproofing and Insulation Works

- The project details shall be compared with the supplier’s details; any differences shall be identified and resolved before the application stage.

- Application details and specifications shall be followed strictly. Deficiencies or defective workmanship in insulation works are very costly and generally impossible to rectify.

Joinery Works

- Detail drawings shall be prepared and any problems that may arise during application shall be resolved at this stage.

- A sample section shall be prepared before application and approved by the supervising engineer.

RECEIVED BY 							NOTIFIED BY

Full Name: 								Full Name:

Signature: 								            Signature:

Date: 								Date: