- The cutting drill bit shall be selected according to the type and thickness of the material to be drilled.

- Always take precautions to ensure that your fingers and hands do not remain in front of the drill; work carefully.

- Ensure that the drill bit is properly seated, centered, securely fixed, and not loose. Do not start work without checking the condition of the drill bit.

- To protect your body and/or hands, hold the hand drill correctly and use its designated handles (to prevent slipping); do not work with the drill too close to your body.

- Do not start work without suitable positioning when there is no adequate support, when you cannot work in a stable and safe manner, or when working on poor surfaces.

- For bench work, secure the hand drill to prevent rotation, slipping, movement of the workpiece, etc.

- Always switch off the hand drill and unplug it when you have finished using it, when changing the drill bit, or when checking any other part.

- When drilling small parts, use a vise; if this is not possible, use a suitable fixing device. The workpiece shall be securely fixed.

- Check that the workpiece fixture and vise are securely fastened to the drill press table.

- Wear safety goggles and a face shield, taking into account that chips may be ejected while drilling.

- Gloves, necklaces, bracelets, rings, or ID tags shall not be worn when working with rotating machinery.

- The sleeves and cuffs of work clothing shall be elasticated; personnel operating a drill shall not wear loose or dangling clothing.

- Ensure that the hand pressing the material against the rotating drill and the other supporting hand remain firm and stable.

- When working on drill presses, wear steel-toe safety footwear to protect against falling parts.

- If excessive heat is generated during drilling, use coolant or oil.

- The machine shall be properly grounded.

ACKNOWLEDGED BY 						NOTIFIED BY

Full Name: 								Full Name:

Signature: 							             Signature:

Date: 									Date: