- The machine must not be operated or used without the supervision of the responsible personnel.

- Before starting and while operating the machine, the operator must ensure that they are at a safe distance from other people and objects.

- Maximum attention must be paid during operation to prevent hand or foot injuries.

- Since the exhaust gases emitted by the machine are toxic, it must not be operated in areas without adequate ventilation.

- Do not refuel while the engine is running. Keep away from open flames.

- Do not operate the machine without wearing the required occupational health and safety equipment.

- Remove stones, wires, pieces of glass, bushes, and metal objects from the work area.

- Do not operate the machine if the safety devices are not in good working condition (Stop/Engine Safety Lever).

- If the machine strikes an object during operation, stop the machine and check for damage.

- Do not operate the machine if the ground is wet.

- Do not operate the machine if the tiller blades are not sharp.

- Do not operate the machine if the Stop/Engine Safety Lever is not functioning.

- If you lose your balance while operating the machine, release the Stop Lever and allow the machine to stop.

- Operate the machine in daylight and in adequately illuminated areas.

- Never leave the machine unattended while it is running.

- Do not operate the machine on excessively steep or sloping terrain that is unsuitable for the machine.

- Do not overfill the fuel tank to avoid the risk of fire.

ACKNOWLEDGED BY 							NOTIFIED BY

Full Name: 									Full Name:

Signature: 							       		Signature:

Date: 										Date: