TRUCK OPERATING INSTRUCTION

- Yükleme sahasında damper kaldırırken, kamyon devrilmesine karşı, zeminin düz olup olmadığına dikkat edilmelidir.

- Excavated material shall never be dumped at the zero point of the slope. Do not approach closer than 7 metres to the edge of the slope.

- When raising the dump body at the loading area, check that the ground is level to prevent the truck from overturning.

- Before raising the dump body, check that it can be raised safely. Pay particular attention to overhead electrical lines, telephone lines, structures, etc.

- The truck shall never be left unattended while it is being loaded.

- Check the vehicle's air and hydraulic systems carefully. If there is a leak, notify the authorized personnel.

- A loaded truck shall never be stopped on a ramp.

- At the loading area, follow the instructions given by the loader operator.

- During loading, ensure that the tailgate doors are closed and that their pins are fully engaged.

- Once the truck is loaded, do not move it until the loader operator has given the signal to proceed.

- There shall be no pedestrians in the loading area; warn them if any are present.

- Never climb onto the truck body in the loading area.

- When the truck is loaded, do not move it without covering the load with the truck body cover.

- The truck body shall be loaded so that the load cannot spill, sag or protrude beyond the body.

- Truck drivers shall NEVER enter between the dump tailgate and the dump body.

- Before moving the truck, make sure that the dump body has been completely lowered.

- Never allow anyone to climb onto the truck.

In the event of even the slightest malfunction in the braking system, the truck shall not be used and the authorized personnel shall be notified.

Compliance with Occupational Health and Safety rules is mandatory during all operations.

RECEIVED BY                                      		DELIVERED BY

Full Name:                                      			Full Name:

Signature:                                      			Signature:

Date:                                           			Date: